Week 3 network labs

Measuring circuit 1: measuring voltages with a multimeter

Use the following values:

R1=470 ohm C1=2.2μF

A0. Measure with an AC meter: U1, UC1 and UR1, and read the AC meter . A1. Do these

measurement data match Kirchhoff's voltage law? Check that you have not made any

measurement errors.

U1 = 29.12 V

UC1=27.66 V I = 19.11 mA

UR1=8.81 V I = 18.74 mA

KVL:

U1^2 = UC1^2+UR1^2

UC1^2+UR1^2 =764.4+77.6=842.0

U1^2 = (29.12)^2 = 848.0

848-842 = 6  this is minimal difference less than 1 percent it matches what we expected.

A1.

Measuring circuit 2: Measuring current with the oscilloscope

B0.

| Ieff() | Imax() | Idmm() | T() |
| --- | --- | --- | --- |
| 36,7mV | 52mV | 108mV | 20.000 ms |

B1.

F=1/T = 1/0,05 = 50HZ

B2. Reactance for capacitors formula =

R1 = 470 Ω

XC1,2 = 1/2pi(50)(2,2x10^-6) = 1447 Ω

Ceq = C1+C2 = 4,4 x10^-6

XCeq =  = 1/2pi(50)(4,4x10^-6) = 723.5 Ω

Total impedance with R1 :

Zceq = −j723.5 Ω

|Z| = square root of (470^2+723,5^2) = 863 Ω

B4. (next page)

Current formula is

I = 29.12/863 =  0,003374 A = 33,74mA

The multi meter is similar.

B5.