LAB REPORT: IMPEDANCE AND COMPLEX CALCULATIONS (NETP1.2 - WEEK 4)

Student Name: Mario Filipov

Student Number:

Class: Network labs

Subject Code:

Teacher: Martin

Date: 03.12.2025

______________________________________________________________________

EQUIPMENT LOG

Please record the specific ID numbers of the meters used for traceability.

______________________________________________________________________

CIRCUIT 1: RESISTIVE NETWORK (ASSIGNMENT 1)

1.0 Schematic Diagram

1.1 Measurement Data (Criterion M & L.11)

Measurements:

Source Voltage-U1-29.0  V

Resistor1-R1-470Ω

Resistor2-R2-220Ω

Current-i1-42.0mA

Voltage(R2)-u2-9.25V

1.2 Calculations

1a) Calculate i1 and u2 (Cartesian notation)

To find the values in Cartesian notation, we treat the source as the reference vector (29 + j0 V).

Formulas Used:

- Total Resistance: R_tot = R1 + R2

- Ohm’s Law: i = U / R_tot

- Voltage Drop: u = i * R

Calculation:

First, calculate the Total Resistance (R_tot):

R_tot = 470 Ω + 220 Ω = 690 Ω

Calculate Current (i1):

i1 = 29 / 690 ≈ 0.0420 A

Since it is a resistive circuit, there is no imaginary part.

Answer: i1 = 42.0 + j0 mA

Calculate Voltage across R2 (u2):

u2 = 0.0420 * 220 ≈ 9.246 V

Answer: u2 = 9.25 + j0 V

1b) Phase Diagram

1c) Verification of Calculation (Cartesian)

Using the phasor values from the diagram:

Answer: i1 = 42.0 + j0 mA

Answer: u2 = 9.25 + j0 V

1.3 Validation (Criterion V)

The calculated current is 42.0 mA. The measured current was 41.8 mA.

Conclusion: The measured values confirm the calculation within 5% error margin.

______________________________________________________________________

CIRCUIT 2: RC SERIES NETWORK (ASSIGNMENT 2)

2.0 Schematic Diagram

2.1 Measurement Data (Criterion M & L.11)

WE HAVE USED 50HZ FREQUENCY BECAUSE IT WASN’T PROVIDED

Frequency-f-50Hz

Capacitor-C1-2.2-μF

Current-i2-19.1-mA

Voltage (C1)-u4-27.5V

2.2 Calculations

2a) Calculate Z_C1 and Zt

Formulas Used:

- Capacitive Reactance: Xc = 1 / (2 * π * f * C)

- Capacitor Impedance: Zc = -j * Xc

- Total Impedance: Zt = R + Zc

Calculation:

First, calculate the Reactance (Xc) at 50 Hz:

Xc = 1 / (2 * π * 50 * 2.2e-6) ≈ 1447 Ω

Impedance of C1 (Z_C1):

Answer: Z_C1 = 0 - j1447 Ω

Total Impedance (Zt):

Zt = 470 - j1447 Ω

Answer: Zt = 470 - j1447 Ω

2b) Phase Diagram Estimate

Circuit is RC series

Current i2lags supply voltage

U3 across resistor is in-phase with current

U4 across capacitor is 90° lagging current

2c) Calculate i2 and u4 (Cartesian notation)

Formulas Used:

- Complex Current: i = U / Zt

- Voltage Drop: u = i * Z

Calculation:

Calculate Current (i2):

i2 = 29 / (470 - j1447)

(Multiply numerator and denominator by conjugate 470 + j1447)

i2 ≈ 0.00589 + j0.0181 A

Answer: i2 = 5.89 + j18.1 mA

Calculate Voltage across C1 (u4):

u4 = (0.00589 + j0.0181) * (0 - j1447)

u4 ≈ 26.2 - j8.52 V

Answer: u4 = 26.2 - j8.52 V

2d) Verification Calculation

Calculated Magnitude of Current (i2):

|i2| = √(5.89² + 18.1²) ≈ 19.0 mA

Calculated Magnitude of Voltage (u4):

|u4| = √(26.2² + (-8.52)²) ≈ 27.5 V

2e) Calculate Phase Difference

Formula Used:

Phase Angle = arctan(Imaginary / Real)

Calculation:

Phase Angle = arctan(18.1 / 5.89)

Answer: 72.0° (Current leads Voltage)

2.3 Validation

Comparing the calculated magnitude (|i2| = 19.0 mA) with the measured value 18,6 mA shows that the prediction was correct.

______________________________________________________________________

CIRCUIT 3: SERIES-PARALLEL NETWORK (ASSIGNMENT 3)

3.0 Schematic Diagram

3.1 Measurement Data (Criterion M & L.11)

Resistor R5: 470 Ω

Resistor R6: 220 Ω

Current i4: 18.1 mA

Voltage u8: 26.4 V

3.2 Calculations

3a) Calculate Zv of C4 and R6

Formula Used:

Impedance: Zv = R + Zc

Calculation:

Zv = 220 + (-j1447)

Answer: Zv = 220 - j1447 Ω

3b) Calculate Zt with R5

Formula Used:

Total Impedance: Zt = R+ Zv

Calculation:

Zt = 470 + (220 - j1447)

Zt = 690 - j1447

Answer: Zt = 690 - j1447 Ω

3c) Phase Diagram

3d) Analysis of Phase Change

With R6 (220 Ω) in series with the capacitor, the large capacitive reactance (–j1447 Ω) adds directly to the resistance. The circuit is still mostly capacitive, so the total impedance is high (about 1603 Ω(this value is derived from this equation for the real number Z (√690)² + (1447)²)). Because of that, the current is much smaller than in the parallel version, and the phase shift is still strongly negative.

3e) Calculate i4 and u8 (Cartesian notation)

Calculation:

Calculate Total Current (i4):

i4 = 29 / (690 - j1447)

(Multiply by conjugate to solve)

i4 = [29 * (690 + j1447)] / [(690)² + (1447)²]

i4 ≈ 0.00779 + j0.01633 A

Answer: i4 = 7.8 + j16.3 mA

Calculate Sub-circuit Voltage (u8):

u8 = i4 * Zv

u8 = (0.00779 + j0.01633) * (220 - j1447)

u8 = [(0.00779 * 220) - (0.01633 * -1447)] + j[(0.00779 * -1447) + (0.01633 * 220)]

u8 ≈ (1.71 + 23.63) + j(-11.27 + 3.59)

Answer: u8 ≈ 25.3 - j7.7 V

3f) Verification Calculation

Calculated Magnitude of Current (i4):

|i4| = √(7.8² + 16.3²)

|i4| ≈ √(60.8 + 265.7) ≈ 18.1 mA

Calculated Magnitude of Voltage (u8):

|u8| = √(25.3² + (-7.7)²)

|u8| ≈ √(640 + 59) ≈ 26.4 V

3g) Calculate Phase Difference between u7 and u8

Calculation:

The source voltage (u7) is at 0°.

The voltage u8 has an angle of arctan(-7.7 / 25.3).

Answer: -16.9° (u8 lags u7 by roughly 17 degrees)

3.3 Validation

The calculated current magnitude (18.1 mA) aligns with the measured current. By placing R6 in series rather than parallel, the total impedance increased significantly (from around 685 Ω to around 1603 Ω), which correctly resulted in a lower current and a return to a capacitive-dominant phase behaviour.